PK/LC/MS Method:

Column:		2.1 X 50 mm, 5 µm , 100 Å , C18

Column Temp:	40 °C

Mobile Phase: 	A = Water, 0.1% Formic Acid

B = Acetonitrile, 0.1% Formic Acid

Flow Rate:		400 µl/min

Gradient:           	Time		%B

(min.)

0		25

2       		80

2.1		25

4            	25

method notes:

a )  Adjust initial %B to suit the compound.

b ) Try reducing the gradient time to one minute to shorten the method

c )  This method was first published at the IonSource.Com web site,  http://www.ionsource.com